Cable Size Chart

Technical Pages

The dimensions below are provided for sizing cable conduit for new units. Older cable dimensions may differ from these, please consult Tech Sheet 88WA2030 – Identifying Power Cables

RHW

(Non-Explosion Proof)

| Cable Size | Jacket OD (mm) | Jacket OD (in) |
| --- | --- | --- |
| 14AWG-4 + 14AWG-4 | 20 | 0.79 |
| 12AWG-4 + 14AWG-4 | 22 | 0.87 |
| 10AWG-4 + 14AWG-4 | 30 | 1.18 |
| 8AWG-4 + 14AWG-4 | 31 | 1.22 |
| 6AWG-4 + 14AWG-4 | 32 | 1.26 |
| 4AWG-4 + 14AWG-4 | 33 | 1.30 |
| 3AWG-4 + 14AWG-4 | 34 | 1.34 |
| 16AWG-12* | 18 | 0.71 |
| 14AWG-4* | 14.5 | 0.57 |

(Non-Explosion Proof)

H07RN8-F Plus (Explosion Proof)

H07RN8-F Plus (Explosion Proof)

| Cable Size | Jacket OD (mm) | Jacket OD (in) |
| --- | --- | --- |
| 14AWG-4 + 14AWG-4 | 20 | 0.79 |
| 12AWG-4 + 14AWG-4 | 22 | 0.87 |
| 10AWG-4 + 14AWG-4 | 30 | 1.18 |
| 8AWG-4 + 14AWG-4 | 31 | 1.22 |
| 6AWG-4 + 14AWG-4 | 32 | 1.26 |
| 4AWG-4 + 14AWG-4 | 33 | 1.30 |
| 3AWG-4 + 14AWG-4 | 34 | 1.34 |
| 16AWG-12* | 18 | 0.71 |
| 14AWG-4* | 14.5 | 0.57 |

| Cable Size | Jacket OD (mm) | Jacket OD (in) |
| --- | --- | --- |
| 4 x 1.5 | 11 | 0.43 |
| 5 x 1.5 | 12 | 0.47 |
| 6 x 1.5 | 14 | 0.55 |
| 7 x 1.5 | 16 | 0.62 |
| 7 x 2.5 | 18 | 0.7 |
| 7 x 4 | 22 | 0.86 |
| 10 x 1.5 | 19 | 0.74 |
| 10 x 2.5 | 21 | 0.82 |
| 10 x 4 | 24 | 0.94 |
| 5 x 6 | 19 | 0.74 |
| 4 x 2.5 | 13 | 0.5 |
| 4 x 4 | 16 | 0.6 |
| 4 x 6 | 18 | 0.7 |
| 4 x 10 | 23 | 0.9 |
| 4 x 16 | 28 | 1.09 |
| 4 x 25 | 32 | 1.25 |
| 4 x 35 | 34 | 1.38 |

*SOOW-type Control Cable

*SOOW-type Control Cable